USER MANUAL

JCAS OFDM Simulation Script v6.0

JCASFasaIMultistaticwith5GAAStandard.m

Galih Nugraha Nurkahfi, M.T.

Pusat Riset Telekomunikasi – BRIN

Jl. Cisitu No. 21/154D, Bandung

Joint Communication and Sensing (JCAS) Research Project

Version 6.0 | March 2026

# 1. Overview

This script implements a full-stack simulation of a Joint Communication and Sensing (JCAS) system using an OFDM waveform in a vehicular (V2X) environment. The simulation is designed to evaluate and compare six subcarrier allocation strategies — three classical and three reinforcement learning-based — under realistic channel and mobility conditions, with all performance metrics benchmarked against the 5GAA WIISAC Intersection Movement Assist (IMA) and Vehicle Sensing for ADAS use case requirements.

The script runs in MATLAB and covers four sequential simulation phases:

- Phase 1 — OFDM waveform generation and radar target returns

- Phase 2 — Hardware impairment modelling (self-interference, CFO, phase noise)

- Phase 3 — Advanced sensing: Range-Doppler processing, CA-CFAR detection, Extended Kalman Filter (EKF) tracking

- Phase 4 — Resource allocation comparison: Static, Water-Filling, Adaptive LoS/NLoS, Q-Learning, SARSA, Greedy Contextual Bandit

The script outputs five interactive debug figures, up to 12 CSV data files, and 11 publication-ready PNG figures per run. When called from the Looping Experiment runner, all outputs are automatically suffixed with the experiment configuration label.

## 1.1  Research Context

This simulation was developed as part of a multi-institutional JCAS research project led by PR Telekomunikasi BRIN, in collaboration with Telkom University Bandung, Telkom University Jakarta, and Institut Teknologi Bandung (ITB). The primary goal is to produce a peer-reviewed journal paper comparing classical and RL-based OFDM subcarrier allocation strategies for vehicular ISAC systems, evaluated against 5GAA WIISAC Service Level Requirements (SLR).

## 1.2  Key Capabilities

- Multistatic sensing topology: Monostatic, Bistatic (Mode B), and Multistatic (Mode C) with RSU + multiple OBU receivers

- 2D EKF tracking with Least Squares triangulation for position estimation

- Three RL algorithms with shared reward function for fair comparison

- Full 5GAA compliance checking on all output KPIs

- Statistical analysis: Bootstrap CI, t-test significance, sensitivity analysis, ablation study

- Automatic export of publication-ready figures and LaTeX-ready data tables

# 2. System Architecture and Script Flow

The script is organised into 12 numbered sections (Bagian 0–11), each handling a distinct processing stage. The diagram below shows the data flow between sections.

| Section | Phase | Function |
| --- | --- | --- |
| Bagian 0 | Setup | Centralised configuration — all cfg.* parameters defined here |
| Bagian 1 | Setup | Node placement, velocity assignment, RCS, lane distribution |
| Bagian 2 | Phase 1 | OFDM waveform generation (IDFT + cyclic prefix + serial concat) |
| Bagian 3 | Phase 1 | ITU-R M.1225 Vehicular A channel convolution (6-tap TDL) |
| Bagian 4 | Phase 1 | Radar channel: multi-target returns + clutter + AWGN |
| Bagian 5 | Phase 2 | Hardware impairments: SI cancellation, CFO compensation, phase noise |
| Bagian 6 | Phase 3 | OFDM demodulation, channel estimation, LMMSE equalisation |
| Bagian 7 | Phase 3 | Range-Doppler map, Chebyshev windowing, 2D FFT |
| Bagian 8 | Phase 3 | 2D CA-CFAR detection, EKF tracking, LS triangulation |
| Bagian 9 | Phase 4 | Resource allocation: Static, Water-Fill, Adaptive, RL (all 3) |
| Bagian 10 | Output | Monte Carlo BER/Pd sweep, SNR vs performance curves |
| Bagian 11 | Output | CRB computation, BW comparison sweep |

After Bagian 11, the script runs: (1) Deep Analysis, (2) Multi-CSV Export, (3) Publication Figures, and (4) Terminal Summary.

# 3. Quick Start

## 3.1  Requirements

- MATLAB R2021b or later

- Required Toolboxes: Communications Toolbox, Phased Array System Toolbox, Sensor Fusion and Tracking Toolbox, Signal Processing Toolbox

- Python 3 with openpyxl installed — only needed if XLSX export is used (CSV export has no Python dependency)

## 3.2  Running the Script

Step 1 — Open MATLAB and set the working directory to the folder containing the script.

cd /path/to/your/project/folder

Step 2 — Open the script and adjust parameters in Bagian 0 (Configuration) as needed (see Section 4).

Step 3 — Run the script from the MATLAB command window or press F5 in the Editor.

JCASFasaIMultistaticwith5GAAStandard

Step 4 — Results are saved automatically to the JCAS_Results/ folder in the working directory.

## 3.3  Expected Runtime

| Configuration | Approx. Runtime | Note |
| --- | --- | --- |
| Default (6 nodes, 20 MHz, Mode C, all RL) | 8–15 min | Depends on CPU speed |
| Deep Analysis disabled | 5–8 min | Set cfg.export.csv = false for deep |
| RL single algorithm (not "all") | 4–6 min | Set cfg.rl.algorithm = "qlearning" |
| BW compare disabled | 6–12 min | Set cfg.BW_compare = false |

Note: The trajectory animation (Figure 5) with 600 frames at 8 fps adds significant time. Set cfg.plot.fig5 = false to skip it during batch runs.

# 4. Configuration Parameters (cfg)

All parameters are defined in Bagian 0 at the top of the script. This is the only section that should be edited for normal use. Parameters are grouped into categories below.

## 4.1  RF and OFDM Parameters

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.fc | 5.9e9 | Hz | Carrier frequency. 5.9 GHz is the ITS-G5 / C-V2X band. |
| cfg.BW | 20e6 | 5e6–40e6 Hz | Total RF bandwidth. Controls range resolution: dR = c/(2B). |
| cfg.BW_compare | true | true ¦ false | If true, runs a separate BW sweep for 10/20/40 MHz. |
| cfg.BW_list | [10e6,20e6,40e6] | array Hz | Bandwidths to sweep when BW_compare = true. |
| cfg.SNR_dB | 15 | -5 to 30 dB | Operating SNR for the primary resource allocation run. |
| cfg.mod_order | 4 | 4 ¦ 16 ¦ 64 | Modulation order: 4=QPSK, 16=16QAM, 64=64QAM. |
| cfg.Nsymb | 64 | 32–256 | OFDM symbols per frame. Affects Doppler resolution. |
| cfg.SNR_sweep | -5:5:25 | dB array | SNR points for the Monte Carlo Pd/BER sweep. |
| cfg.Ntrials | 30 | 10–100 | Monte Carlo trials per SNR point. 30 is statistically sufficient. |

## 4.2  Road Geometry

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.road.n_lanes_per_dir | 3 | 1–4 | Lanes per direction. Total lanes = 2× this value. |
| cfg.road.lane_width_m | 3.5 | 3.0–4.0 m | Standard Indonesian lane width. |
| cfg.road.length_m | 300 | 100–600 m | Road segment length. Affects node spacing. |
| cfg.road.max_detect_m | 200 | 50–250 m | Max sensing range. 5GAA infra-based limit is 250 m. |

## 4.3  Multistatic Topology

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.multi.mode | "C" | "mono"¦"A"¦"B"¦"C" | Sensing topology. See table below for details. |
| cfg.multi.obu_node_idx | [1,4,7] | scalar or array | Node indices that act as OBU receivers. Mode B: scalar. Mode C: array. |

| Mode | Configuration | Description |
| --- | --- | --- |
| "mono" | 1 RSU only | Monostatic. RSU at road centre. Baseline for comparison. |
| "A" | 2 RSUs | Two RSUs at road ends. No vehicle-based receivers. |
| "B" | 1 RSU + 1 OBU | Bistatic. One vehicle acts as passive receiver. obu_node_idx = scalar. |
| "C" | 1 RSU + N OBUs | Multistatic. Multiple vehicles as receivers. obu_node_idx = array. Recommended for paper. |

## 4.4  Vehicle Mobility

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.v2x.Nnodes | 6 | 2–20 | Number of traffic participants in the scenario. |
| cfg.v2x.mobility_model | "urban" | "urban"¦"highway"¦"mixed" | Mobility profile. Affects RCS distribution and node type mix. |
| cfg.v2x.v_mean | 14 | 0–25 m/s | Mean node velocity. 14 m/s ≈ 50 km/h (urban). 5GAA max: 16.67 m/s. |
| cfg.v2x.v_std | 4 | 0–10 m/s | Velocity standard deviation. |
| cfg.v2x.Nframes_track | 600 | 100–2000 | EKF tracking frames. More frames = smoother RMSE estimate. |
| cfg.manual_targets | false | true ¦ false | If true, overrides mobility model with manual target definitions. |

## 4.5  Hardware Impairments (Phase 2)

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.hw.SI_dB | -15 | dBc | Self-interference power before cancellation. |
| cfg.hw.SI_cancel_dB | 20 | 10–40 dB | LMS cancellation depth. Residual SI = SI_dB - SI_cancel_dB. |
| cfg.hw.CFO_ppm | 10 | 0–50 ppm | Carrier frequency offset. 10 ppm matches USRP B210 spec. |
| cfg.hw.phase_noise_dBc | -80 | dBc/Hz | Phase noise at 1 kHz offset. |

## 4.6  CFAR Detection (Phase 3)

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.cfar.Pfa | 1e-3 | 1e-4 to 1e-1 | Design false alarm probability. 1e-3 is more conservative than 5GAA 1% budget. |
| cfg.cfar.guard_cells | 2 | 1–4 | Guard cells in 2D CA-CFAR. Protects against target self-masking. |
| cfg.cfar.train_cells | 8 | 4–16 | Training cells. More cells = better noise estimate, slower computation. |
| cfg.window.type | "chebyshev" | "chebyshev"¦"hamming"¦"hann" | Range-Doppler window type. |
| cfg.window.cheby_att | 60 | 40–80 dB | Chebyshev sidelobe attenuation in dB. |

## 4.7  Resource Allocation (Phase 4)

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.ra.ratios_sweep | 0.1:0.1:0.9 | array | Subcarrier ratios rho to sweep for all 6 strategies. |
| cfg.ra.min_throughput | 2e6 | bps | Minimum throughput constraint for feasibility check. Note: 5GAA SLR is 0.2 Mbps = 2e5 bps. |
| cfg.ra.LoS_SNR_thresh | 10 | dB | SNR threshold to classify current channel as LoS or NLoS. |
| cfg.load.interference_factor | 0.3 | 0–1 | Channel load interference scaling factor for RL state computation. |
| cfg.load.occupancy_thresh | 0.5 | 0–1 | Occupancy threshold for busy/idle channel classification. |

## 4.8  Reinforcement Learning

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.rl.algorithm | "all" | "all"¦"qlearning"¦"sarsa"¦"bandit" | Which RL algorithms to train and evaluate. "all" runs all three. |
| cfg.rl.n_episodes | 3000 | 500–10000 | Training episodes per algorithm. |
| cfg.rl.alpha | 0.1 | 0.01–0.5 | Q-table learning rate. |
| cfg.rl.gamma | 0.9 | 0.5–0.99 | Discount factor for future rewards. |
| cfg.rl.epsilon | 0.3 | 0.1–0.9 | Initial exploration rate for epsilon-greedy policy. |
| cfg.rl.epsilon_decay | 0.995 | 0.99–0.9999 | Multiplicative epsilon decay per episode. |
| cfg.rl.w_comm | 0.4 | 0–1 (sum to 1) | Reward weight for communication throughput component. |
| cfg.rl.w_sense | 0.4 | 0–1 (sum to 1) | Reward weight for sensing RMSE quality component. |
| cfg.rl.w_detect | 0.2 | 0–1 (sum to 1) | Reward weight for detection probability component. |

The three reward weights (w_comm + w_sense + w_detect) should sum to 1.0. Changing these weights is the primary sensitivity analysis experiment.

## 4.9  Export Settings

| Parameter | Default Value | Valid Range | Description |
| --- | --- | --- | --- |
| cfg.export.figures | true | true ¦ false | Save all publication figures as PNG 300 dpi to export_dir. |
| cfg.export.csv | true | true ¦ false | Save all 12 CSV data files and run deep analysis. |
| cfg.export.dir | "JCAS_Results" | string | Output folder name. Created automatically if it does not exist. |
| cfg.plot.fig1 | true | true ¦ false | Show Figure 1: Sensing Core (Range-Doppler, CFAR, Pd vs SNR). |
| cfg.plot.fig2 | true | true ¦ false | Show Figure 2: JCAS Tradeoff (Pareto, BER, RL comparison). |
| cfg.plot.fig3 | true | true ¦ false | Show Figure 3: Sensing Summary (velocity, RMSE, KPI table). |
| cfg.plot.fig4 | true | true ¦ false | Show Figure 4: Multistatic analysis (geometry, triangulation). |
| cfg.plot.fig5 | true | true ¦ false | Show Figure 5: Trajectory animation (600 frames, slow — disable for batch runs). |

# 5. Experiment Variables

The parameters below are the most commonly changed between experiment runs. They directly affect the research findings reported in the paper.

| Experiment Type | Parameter to Change | Example Values |
| --- | --- | --- |
| BW Sweep | cfg.BW | 10e6 → 20e6 → 40e6 (3 separate runs) |
| Density Sweep | cfg.v2x.Nnodes | 4 → 6 → 10 (3 runs matching 5GAA low/medium/high) |
| Topology Sweep | cfg.multi.mode | "mono" → "B" → "C" (3 runs) |
| SNR Sweep | cfg.SNR_sweep | Already automatic: -5:5:25 dB per run |
| RL Ablation | cfg.rl.w_comm, cfg.rl.w_sense, cfg.rl.w_detect | Deep analysis runs this automatically |
| Single Algorithm | cfg.rl.algorithm | "qlearning" ¦ "sarsa" ¦ "bandit" (for faster debug runs) |
| Operating SNR | cfg.SNR_dB | 5, 10, 15, 20 (reporting point on SNR curve) |

For batch runs across all experiment variants, use the companion script JCASFasaIMultistaticwith5GAAStandard_LoopingExperiment.m which sets these parameters automatically.

# 6. Output Files

## 6.1  Folder Structure

Working Directory/

JCAS_Results/

CSV01_Paper_Summary[suffix].csv

CSV02_Pareto_Sweep[suffix].csv

CSV03_Pd_vs_SNR[suffix].csv

CSV04_PerNode_RMSE[suffix].csv

CSV05_BW_Comparison[suffix].csv

CSV06_RL_Training[suffix].csv

CSV07_Statistical_Significance[suffix].csv

CSV08_Sensitivity_Analysis[suffix].csv

CSV09_Convergence[suffix].csv

CSV10_Ablation[suffix].csv

CSV11_Generalization[suffix].csv

CSV12_System_Config[suffix].csv

Fig1_Sensing_Core.png

Fig2_JCAS_Tradeoff.png

Fig3_Sensing_Summary.png

Fig4_Multistatic.png

PubFig1_Pareto.png  ...  PubFig11_CRB_vs_EKF.png

## 6.2  CSV Files

| File | Contents |
| --- | --- |
| CSV01 — Paper Summary | Main results table per strategy. PASS/FAIL per 5GAA KPI. Ready to copy into paper. |
| CSV02 — Pareto Sweep | Capacity and RMSE for every rho value (0.1–0.9) for all 6 strategies. |
| CSV03 — Pd vs SNR | Detection probability, BER (simulated and theoretical), Pfa per SNR point from Monte Carlo. |
| CSV04 — Per-Node RMSE | Raw and EKF RMSE per individual node, target type, RCS, range, speed, lane. |
| CSV05 — BW Comparison | Range resolution, CRB, and RMSE for each bandwidth in BW_list. |
| CSV06 — RL Training | Reward per episode (sampled every 10 episodes) for all RL algorithms. |
| CSV07 — Statistical Sig. | Bootstrap 95% CI and t-test p-value vs Static for each strategy. |
| CSV08 — Sensitivity | Best-feasible capacity vs reward weight w_c sweep for Q-Learning and SARSA. |
| CSV09 — Convergence | Moving-average reward (window 100) per episode for each RL algorithm. |
| CSV10 — Ablation | Capacity under 4 reward configurations (full, no detection, no sensing, no comm). |
| CSV11 — Generalization | Q-Learning tested on low/medium/high density without retraining. |
| CSV12 — System Config | All simulation parameters for reproducibility documentation. |

## 6.3  Publication Figures (PubFig)

| Figure | Contents and Paper Use |
| --- | --- |
| PubFig1 — Pareto | Primary result. Sensing RMSE vs communication capacity for all 6 strategies with 5GAA reference lines. |
| PubFig2 — Bar KPI | Bar chart of best-feasible capacity and Pd at operating SNR per strategy. |
| PubFig3 — Pd/BER vs SNR | Detection probability and BER vs SNR curves with theoretical BER overlay. |
| PubFig4 — BW Comparison | Range resolution and EKF RMSE vs bandwidth (dual y-axis). |
| PubFig5 — RL Convergence | Moving-average reward vs episode for all RL algorithms. |
| PubFig6 — Bootstrap CI | Bar chart with 95% confidence interval error bars per strategy. |
| PubFig7 — Sensitivity | Best-feasible capacity vs reward weight w_c for Q-Learning and SARSA. |
| PubFig8 — Ablation | Bar chart of capacity under different reward component configurations. |
| PubFig9 — Generalization | Q-Learning capacity under low/medium/high density without retraining. |
| PubFig10 — Per-Node RMSE | Grouped bar chart of raw vs EKF RMSE per node, coloured by target class. |
| PubFig11 — CRB vs EKF | EKF RMSE vs SNR compared to the Cramer-Rao Lower Bound. Shows theoretical gap. |

All PubFig files are saved at 300 dpi with white background and are suitable for direct submission to IEEE journal figures. File size is typically 300–600 KB per figure.

# 7. 5GAA Compliance Reference

The table below maps each 5GAA WIISAC IMA SLR to the corresponding simulation parameter and where the PASS/FAIL result appears in the output.

| 5GAA KPI | Target | Script Parameter | Output Location |
| --- | --- | --- | --- |
| Sensing range (infra) | ≤ 250 m | cfg.road.max_detect_m = 200 | CSV12, terminal summary |
| Range resolution | ≤ 1 m | cfg.BW (controls delta_R = c/2B) | CSV01, CSV05, PubFig4, terminal |
| Sensing confidence | ≥ 95% | cfg.Ntrials = 30 (MC trials) | CSV03, PubFig3, terminal |
| Missed detection | ≤ 1% | (1 - Pd_arr) × 100 | CSV03 |
| False alarm rate | ≤ 1% | cfg.cfar.Pfa = 1e-3 (design) | CSV03, terminal |
| Speed accuracy | ≤ 0.3 m/s | delta_v computed from BW, Nsymb | CSV12 |
| Min. throughput | ≥ 0.2 Mbps | cfg.ra.min_throughput in reward | CSV01, CSV02, PubFig2 |

# 8. Troubleshooting

| Error / Symptom | Cause and Fix |
| --- | --- |
| Unrecognized function: deal_ternary | Script called without running to completion first. Make sure the full script runs so local functions are loaded. This error does not occur when run normally from the Editor. |
| fopen returns -1 / fprintf invalid file identifier | Export directory cannot be created. Check that cfg.export.dir is a valid path and MATLAB has write permission to the working directory. |
| CFAR detects nothing (Pd = 0) | SNR is too low for the current Pfa setting. Try lowering cfg.cfar.Pfa (e.g. to 0.01) or raising cfg.SNR_dB. Also check cfg.clutter.CNR_dB — high clutter raises the noise floor. |
| RL capacity lower than Static | Reward weights need adjustment, or training episodes are insufficient. Try cfg.rl.n_episodes = 5000 and check CSV09 convergence plot to confirm the algorithm has converged. |
| Figure 5 animation very slow | Set cfg.v2x.Nframes_track = 200 to shorten the animation, or set cfg.plot.fig5 = false to skip it entirely during batch runs. |
| Python / XLSX not generated | openpyxl is not installed. Run: pip install openpyxl. Alternatively, all data is already saved in the 12 CSV files which do not require Python. |
| EKF RMSE very high (> 10 m) | EKF initialisation noise too large for the current delta_R. Increase cfg.BW to improve range resolution, or reduce cfg.v2x.v_std to reduce velocity uncertainty. |
| Loop runner: variables lost after run() | The base script ran clear. Make sure loop_override = true is set before calling run(). The base script skips clear when called from the runner. |

# 9. Quick Reference Card

This page summarises the most important settings for common use cases.

## Primary Experiment (Paper Default)

cfg.BW = 20e6;              % 20 MHz

cfg.v2x.Nnodes = 6;         % 6 nodes (5GAA IMA baseline)

cfg.multi.mode = "C";        % RSU + 3 OBUs

cfg.SNR_dB = 15;            % Operating SNR

cfg.rl.algorithm = "all";   % Train all 3 RL algorithms

cfg.export.csv = true;       % Generate all CSVs including deep analysis

cfg.export.figures = true;   % Generate all 11 PubFigs

## Fast Debug Run (Skip Animation and Deep Analysis)

cfg.plot.fig5 = false;       % Skip trajectory animation

cfg.export.csv = false;      % Skip deep analysis (bootstrap, ablation)

cfg.rl.algorithm = "qlearning"; % Train only Q-Learning

cfg.Ntrials = 10;           % Fewer MC trials for speed

## BW Sweep Experiment

% Run 3 times, changing cfg.BW each time:

cfg.BW = 10e6;  % Run 1

cfg.BW = 20e6;  % Run 2 (primary)

cfg.BW = 40e6;  % Run 3

## Topology Sweep Experiment

% Run 3 times, changing cfg.multi.mode:

cfg.multi.mode = "mono";  % Run 1: monostatic

cfg.multi.mode = "B";     % Run 2: RSU + 1 OBU (bistatic)

cfg.multi.mode = "C";     % Run 3: RSU + 3 OBUs (multistatic)

## Batch Run (All Experiments Automatically)

% Run the looping experiment script instead:

JCASFasaIMultistaticwith5GAAStandard_LoopingExperiment

The LoopingExperiment script runs all 7 experiment configurations automatically, generates comparison figures, and produces the two LaTeX sections for the paper.